Table of Contents

# IoT Signal Inventory — Kraal Code

Kraal Code integrates a small set of operational-technology peripherals. This inventory enumerates each device signal, its transport, direction, frequency, and the consuming module. It is the signal-inventory artifact required under ISO/IEC/IEEE 42010:2011 §5.5.

## In-Scope Signals (Phase 1)

| Signal | Source device | Transport | Direction | Frequency | Consuming module |
| --- | --- | --- | --- | --- | --- |
| Net weight reading | Weighbridge / platform scale indicator | RS-232 serial (configurable baud, frame, parse pattern) | Inbound | On capture request | Outgrower & Farmer Operations (intake) |
| Stability flag | Weighbridge indicator | RS-232 serial (configurable byte position; default none) | Inbound | With each weight frame | Outgrower & Farmer Operations (intake) |
| Barcode / QR scan | Handheld or camera scanner | USB-HID / keyboard wedge | Inbound | On scan | Inventory, POS, Asset verification |
| Thermal receipt print job | Thermal receipt printer | ESC/POS over USB or network | Outbound | Per transaction | POS |
| Biometric attendance event | Biometric terminal (device-agnostic adapter) | Vendor SDK / device API (configurable) | Inbound | On clock-in/out | HR & Payroll (attendance) |

## Out-of-Scope (Roadmap)

Vehicle telematics and live fleet sensor streams are owned by the Transportation & Fleet module, which is Roadmap (see the feature register). Direct meter telemetry ingestion for asset condition-based maintenance is a Roadmap extension of the Asset Management module; Phase 1 captures condition events through manual inspection and mobile meter entry.

## Signal Handling Principles

- All device interfaces are hardware-agnostic: serial parameters, parse patterns, and device adapters are tenant configuration, not hard-coded per model.

- Inbound signals are validated and, where they create financial or inventory effect (weighbridge intake), are recorded with operator confirmation and an audit trail entry.